Chapter VI

Lead-Based Paint (LBP)

Requirements

## I.  Introduction

The purpose of this chapter is to provide information and guidance on the Federal lead-based paint (LBP) regulations. HUD lead rules apply to housing constructed before January 1, 1978. (The manufacture of lead-based paint was banned in 1978.)

The appendix of this chapter has these selected portions of the HUD Lead Safe Housing Rule, and other general parts of the rule, Within this chapter, you will find the LBP requirements by activity, a Q&A section, and some useful appendixes items that will hopefully support you program successfully implement HUD’s Lead Safe Housing Rule.

Recipient Responsibilities

All HOME Recipients are required to follow the HUD Lead Safe Housing Rule as listed in 24 CFR Part 35 in implementing their activities.  Part J is for Housing Rehabilitation Programs, part K is for Homebuyer Assistance Programs, and park M relates to Tenant Based Rental Assistance.  Essentially, HOME Recipients are responsible to inform residents of the potentials of LBP hazards in their home, evaluate the degree of LBP hazards, mitigate these hazards, provide clearance on the rehabilitated areas affected by the LBP work, and provide all appropriate notices.

Sanctions – Recipients and sub-recipients of HOME funds that fail to comply with HUD’s lead requirements are subject to available sanctions under the HOME program, and may be subject to other penalties authorized by law. Giving notice of a possible LBP hazard does not relieve a property owner of the responsibility for evaluating and reducing lead hazards

## Importance of Addressing Lead-Based Paint

Deteriorated LBP, as well as lead-contaminated dust and soil, are the most common sources of exposure to lead.  LBP requirements have become more stringent over the years due to increased understanding of the harmful effects of lead exposure on children and adults.  Young children are particularly vulnerable to lead poisoning.  A large percentage of children with lead poisoning are in low-income families living in older homes with heavy concentrations of LBP.

Lead poisoning can cause permanent brain damage, reduced intelligence, and behavior problems in children, as well as other health problems in both children and adults.

HOME Recipients and other parties can face lawsuits for failing to address LBP hazards in a unit if a young child is poisoned.  By taking action to reduce lead hazards, contractors can demonstrate that they are working to provide safe, suitable housing, thus reducing liability risks HOME Recipients who take action to reduce lead hazards may also be able to get more favorable insurance premiums.

## II. GENERAL REQUIREMENTS

Major Exemptions (24 CFR 35.115)

HUD’s LBP requirements do not apply to:

- Dwellings completed on or after 1/1/78,

- Housing exclusively for the elderly or person with disabilities, unless a child under age six resides or is expected to reside there (100 days or longer).

- Zero bedroom dwellings, including efficiency apartments, single-room occupancy housing, dormitories, or military barracks.

- Housing found by certified inspection to be free of LBP,

- Housing in which all LBP has been properly identified, removed, and cleared (This does not apply where enclosure or encapsulation has been used as a method of abatement),

- Unoccupied reconstruction housing that will remain vacant until it is demolished to the foundation,

- Non-residential property:  Property or part of a property that will not be used for human residential habitation.  Not exempt are common use areas, such as entryways, hallways, corridors, passageways, stairways or building exterior in a mixed-use building.

- Rehabilitation that does not disturb a painted surface,

- Emergency repair action, which are those needed to safeguard against imminent danger to human life, health or safety, or to protect property from further structural damage, and

- Historical sites/dwellings may use interim controls instead of abatement, under certain conditions.

III. Working Safely with Lead (24 CFR 35.1350)

HUD’s LBP regulations establish safe work practices which must be followed at all times. The exception to this rule is if the painted area to be affected falls within the de minimis levels.  These are:

- 20 sq. ft.(2 sq. meters) on exterior surfaces

- 2 sq. ft. (0.2 sq. meters) in any one interior room or space, or

- 10 percent of the total surface area on an interior or exterior type of component with a small surface area.  Examples include window sills, baseboards, and trim.

The intent of work safe methods is to minimize the spread of lead paint dust, paint chips, and debris.  The following are the major provisions in the safe work practices regulations:

### Qualifications to Perform Safe Work Practices

Workers must be trained in Safe Work Practices.  For additional information on this training, contact: http://www.leadlisting.com/

Occupant Protection

The occupant and the environment must be protected from lead-contaminated or lead-containing materials during hazard reduction activities.  The areas of concern are:

  - Occupants at worksite:  Occupants shall not be permitted to enter the worksite during hazard reduction activities until after hazard reduction work has been completed and clearance obtained, if required.

- Protection of Occupants Belongings:  The dwelling and worksite shall be secured against unauthorized entry, and occupants’ belongings shall be protected from contamination during hazard reduction activities by relocating or covering and sealing them.

- Temporary Relocation:  Occupants shall be temporarily relocated before and during hazard reduction activities to a suitable, decent, safe, and similarly accessible unit free of LBP hazards, except if any one of the following are true:

- Treatment will not disturb LBP, dust-lead hazards or soil lead hazards.

- Interior:  Treatment of the interior will be completed within one period in eight daytime hours, the site will be contained, and the work will not create other safety, health, or environmental hazards.

- Building exterior:  The windows, doors, ventilation intakes and other openings near the worksite are sealed during hazard control work and cleaned afterward; and a lead free entry is provided.

- Treatment will be completed within five calendar days; the work area is sealed; the area within 10 feet of the containment area is cleared of debris at the end of the day; occupants have safe access to sleeping areas, bathroom, and kitchen facilities; and treatment does not create other safety, health, or environmental hazards.

Worksite Preparation and Containment

The worksite for lead hazard reduction activities must be prepared to prevent the release of leaded dust and debris.

Workers must use practices that minimize the spread of leaded dust, paint chips, soil, and debris.

Warning signs are required at each entry to a room where lead hazard reduction activities are conducted when occupants are present, at the main and secondary entryways to a building from which occupants have been relocated, and at exterior worksites at a size and type readable from 20 feet (six meters) from the edge of the worksite.  Signs need to be in the occupants’ primary language to the extent practicable.

Prohibited Work Methods

The methods identified below may not be used at any time for work on surfaces known or suspected to contain LBP.

- Open flame burning or torching.

- Machine sanding or grinding without a high-efficiency particulate air (HEPA) local exhaust control.

- Abrasive blasting or sandblasting without HEPA local exhaust control.

- Heat guns operating above 1,100 degrees Fahrenheit, or those that operate high enough to char the paint.

- Dry sanding or dry scraping.

- Paint stripping in a poorly ventilated space using a volatile stripper that is a hazardous substance in accordance with regulations

Worksite Cleanup

Good cleanup is critical to passing clearance and leaving the unit safe for habitation.  The worksite shall be cleaned using methods, products, and devices that are successful in cleaning lead-contaminated dust, such as vacuum cleaners with HEPA filters or equivalent equipment and household or lead specific detergents or equivalent products.

Safe Work Practice Exemptions

- Safe work practices are not required if paint has been tested and found to be lead-free.

- Safe work practices are not required in houses completed after 1978.

Clearance (24 CFR 35.1340)

Clearance is performed to determine whether the lead hazard reduction process is complete and that no lead-dust or soil hazards remain in the areas of concern. The clearance report requires DHS Form 8552 (see Appendix VI-E at the end of this Chapter).  A clearance examination involves a visual assessment, and dust and soil testing to determine if the unit is safe for occupancy.  A certified inspector/risk assessor must perform clearance.

The clearance examiner must prepare a clearance report in accordance with (24 CFR Part 35.1340) if lead hazard reduction activities other than abatement are performed.  Use DHS form 8552.  If abatement is conducted, a certified supervisor or project designer must prepare an abatement report in accordance with 40 CFR 745.227(e) (10).  Essentially, this requires DHS form 8551 (see Appendix VI-F at the end of this Chapter), which should have been previously filled out, to be attached to the clearance form (DHS 8552).  The clearance notice must note each failed clearance.

IV. LEAD HAZARD EVALUATION (24 CFR 35.110 and 35.1320)

- Lead Hazard Evaluation methods involve an examination of a dwelling to check for lead hazards for every activity.  Evaluation methods include risk assessments, lead hazard screenings, visual assessments, presumption of LBP, and paint testing.

- In California, the Department of Health Services certifies workers, supervisors, inspectors, and risk assessors to perform lead evaluation.  Refer to the Department’s website for training information; http://www.dhs.ca.gov/childlead/. Below is a brief description of each evaluation method.

Risk Assessment

Risk Assessment is a comprehensive investigation of a dwelling to identify LBP hazards. Risk Assessment includes paint testing, dust and soil sampling, and a visual evaluation.  Risk assessment results are summarized in a written report with recommendations for actions.  Risk assessments are conducted by inspectors/risk assessors certified by the Department of Health Services.

Lead Hazard Screening

Lead Hazard Screening is similar to a risk assessment.  While the sampling is less extensive, the requirements are more stringent.  If LBP hazards are detected, a full risk assessment must then be conducted.  Lead hazard screens are conducted by certified risk assessors.

Visual Assessment

A Visual Assessment of deteriorated paint consists of a visual search for cracking, scaling, peeling, or chipped paint.  A visual assessment does not require notification of lead hazard evaluation since the assessment does not evaluate lead-based paint and/or lead hazards.  However, if hazards are discovered and addressed, the contractor must still meet the hazard reduction activity notification requirement.  Visual assessments can be conducted by those who have taken the HUD online test at: http://www.hud.gov/offices/lead/training/

Presumption

Presumption is an administrative decision, and no formal training is required. It makes a determination of LBP and/or LBP hazards, based on non-testing information, and is as an alternative to performing lead hazard evaluation activities.  In some cases, by presuming LBP and/or LBP hazards, hazard evaluations may not be required, but contractors must still conduct lead hazard reduction activities as required.

Paint Testing

Paint Testing entails testing painted surfaces to determine if they contain LBP using methods such as an x-ray fluorescence (XRF) analyzer or laboratory analysis.  Paint testing must be conducted by certified inspectors/risk assessors.

V.  LEAD HAZARD REDUCTION METHODS

(24 CFR 35.1330, 35.1325, 35.1335)

- Lead hazard reduction methods refer to specific types of treatment to address LBP hazards.  Nothing precludes contractors from conducting additional lead hazard reduction methods beyond the minimum established for each activity.  Lead hazard reduction methods include:

Interim Controls

Interim controls temporarily reduce exposure to LBP hazards through repairs, painting, maintenance, special cleaning, occupant protection measures, clearance, and education programs. A detailed description of interim controls can be found in 24 CFR 35.1330. Interim Controls may be triggered in rehabilitation projects receiving more than $5,000 per unit.

Interim control methods include, but are not limited to:

- Paint stabilization.  Repairing any physical defect in the substrate of a painted surface that is causing paint deterioration, removing loose paint and other material from the surface to be treated, and applying a new protective coating or paint.

- Treatment for friction and impact surfaces.  If LBP is found and exceeds acceptable levels or is presumed, the conditions creating friction or impact with surfaces with LBP such as those that rub, bind, or crush must be corrected.  Examples of this work include hanging/binding doors, installing doorstops, or reworking windows.

- Safe work practices:  All interim controls shall incorporate the use of safe work practices.

- Treatment for chewable surfaces.  If a child under age six has chewed surfaces known to contain LBP or if LBP is presumed, these surfaces must be enclosed or coated, so they are impenetrable.

- Lead-contaminated dust control.  All horizontal surfaces that are rough, pitted or porous such as bare floors, stairs, window sills, and window troughs must be covered with a smooth, cleanable covering or coating such as metal coil stock, plastic, polyurethane, or linoleum.  Carpeting must be vacuumed, or rugs must be removed and vacuumed on both sides.  Vacuuming must be done using HEPA vacuums.

- Lead-contaminated soil control.  If soil is lead-contaminated, interim controls that may be used include impermanent surface coverings such as gravel, bark, and sod as well as land use controls such as fencing, landscaping, and warning signs.

Standard Treatments (24 R 35.1335)

Under certain conditions, if LBP is presumed, then the standard treatment method is triggered.  Standard treatments apply to all applicable surfaces, including soil, to control LBP hazards that may be present.  These methods include:

- Paint Stabilization:  All deteriorated paint on exterior and interior surfaces must be stabilized through repairs, safe paint removal, and repainting.  Or abatement may be performed.

- Smooth and Cleanable Horizontal Surfaces:  All horizontal surfaces that are rough, pitted, or porous such as bare floors, stairs, windowsills, and window troughs must be covered with a smooth, cleanable covering or coating such as metal coil stock, plastic, polyurethane, or linoleum.

- Correcting Dust-Generating Conditions:   All conditions that generate lead-contaminated dust such as those that rub, bind, or crush surfaces with LBP must be corrected.  Examples include re-hanging doors, installing doorstops, or reworking windows.

- Bare Residential Soil:  Soil is addressed using interim control methods including impermanent surface covering such as gravel, bark, and sod as well as land use controls such as fencing, landscaping, and warning signs.

- Safe work practices:  All standard treatments shall incorporate the use of safe work practices.

- Clearance:  A clearance examination shall be performed at the conclusion of lead hazard reduction activities.

- Qualifications:  An individual performing standard treatment must meet the training and/or supervision requirements.

Abatement (24 CFR 35.1325)

Abatement permanently (for at least 20 years) removes LBP and LBP hazards by a variety of measures, including removing LBP and its dust, encapsulating or enclosing the LBP, replacing components containing LBP, or removing or covering lead-contaminated soil.  Abatement may be triggered in rehabilitation projects receiving more than $25,000 per unit. All abatement methods shall incorporate the use of safe work practices.  Abatement shall then be conducted on all applicable surfaces, including soil, and completed when clearance is achieved.

Qualifications to Perform Lead Hazard Reduction

Paint Stabilization, Interim Controls, and Standard Treatments require certification as workers or supervisors. Abatement must be conducted by certified workers and supervisors certified by California’s Department of Health Services.

VI. REHABILITATION

Requirements in the regulations for rehabilitation activities are found in 24 CFR Part 35, Subpart J.  Lead hazard evaluation and reduction activities must be carried out in the rehabilitation of housing constructed prior to 1978.

Federally Rehabilitation Assistance categories  (24 CFR 35.930)

A.  Rehabilitation Projects Less Than or Equal To $5000

Rehabilitation of residential property receiving an average of up to and including $5,000 per unit in Rehabilitation Assistance are required to complete the following:

- Lead Hazard Evaluation   Paint testing must be conducted to identify lead based paint on all painted surfaces that will be disturbed or replaced.   However, the HOME Recipient may presume that LBP exists on all painted surfaces that will be disturbed or replaced and skip paint testing.

- Lead Hazard Reduction.  HOME Recipient must repair all paint that will be disturbed during rehabilitation.  If LBP is detected or assumed, safe work practices must be used during rehabilitation.

- Noticing

- Clearance is required only for the work area.

B.  Rehabilitation Projects:  Over $5,000 to $25,000 Per Unit

In general, for these projects, paint testing and a risk assessment must be conducted to identify lead-based paint on all painted surfaces that will be disturbed or replaced. A risk assessment must be conducted prior to rehabilitation to find LBP hazards in assisted units, in common areas that service those units, and on exterior surfaces. Interim controls should be used to reduce the lead hazards unless the presence of LBP is presumed.

However, there are four other options for determining the level of lead hazard:

(1)  Presume that LBP hazards exist on all painted surfaces to be disturbed or replaced and use interim controls.  A risk assessment is still required.

(2) Presume that LBP hazards exist on all painted surfaces. In this instance Standard Treatments must be performed and a risk assessment is not required.

(3) Conduct paint testing on all non-intact paint surfaces.  If no LBP is detected, then no interim controls are required on that surface.  A risk assessment is still required.

(4) Conduct a lead hazard screen instead of a risk assessment.  If the lead hazard screen indicates that there is no lead contamination, no lead hazard reduction is required.  If the lead hazard screen indicates the presence of lead levels that exceed interior lead dust standards, then a risk assessment must be conducted.

Noticing and clearance of Lead Hazards is always required for these projects.

C.  Rehabilitation Projects Over $25,000 Per Unit

In general for these projects, paint testing and a risk assessment must be conducted to identify lead-based paint on deteriorated painted surfaces or surfaces that will be disturbed or replaced.  A risk assessment must be conducted prior to rehabilitation to find LBP hazards in assisted units, in common areas that service those units, and on exterior surfaces, or HOME

However, there are three other options for determining the level of lead hazard:

(1)  Presume that LBP hazards exist on all painted surfaces to be disturbed or    replaced and use interim controls Abate these surfaces.

(2)  Presume that LBP hazards exist on all painted surfaces.  Abatement is required on these surfaces.  In such cases, evaluation is not required.

(3) The HOME Recipient is permitted to conduct a lead hazard screen instead of conducting a risk assessment.  If the lead hazard screen indicates that there is no lead contamination, no lead hazard reduction is required.  If the lead hazard screen indicates that lead is present, then a risk assessment must be conducted.

Lead Hazard Reduction in projects over $25,000 per unit

Abatement must be completed to permanently reduce LBP hazards if LBP hazards are detected during the evaluations on interior surfaces in the dwelling units and the common areas that service those units or on exterior surfaces, including soil, to be disturbed by rehabilitation,

Interim controls may be completed instead of abatement if LBP hazards are detected on the exterior surfaces that are not disturbed by rehabilitation

Noticing and clearance of Lead Hazards is always required for these projects.

D. Calculating the Level of Federal Rehabilitation Assistance (24 CFR 35.930)

The lead hazard evaluation and reduction activities required for rehabilitation projects depend on the level of Federal Rehabilitation Assistance (FRA) projected for each project.  The FRA breaks up into three categories:

- FRA of up to and including $5,000 per unit,

- FRA of more than $5,000 per unit, up to and including $25,000 per unit,

- FRA of more than $25,000 per unit.

The FRA is determined by comparing the per-unit rehabilitation hard costs and the overall per unit Federal Assistance.  The lower of these calculations is used as the FRA amount.

Hard Costs:  Rehabilitation hard costs are calculated using the actual costs associated with the physical development of a unit, regardless of the source of these funds. (These do not include soft costs, such as administration, relocation, environmental review, and acquisition costs or financing fees, credit reports, title binders and insurance, recordation fees, transaction taxes, impact fees, legal and accounting fees, appraisals, and architectural and engineering fees.)

Federal Assistance:  Federal Assistance is determined by tabulating all Federal funds provided to the project for housing assistance.  This also includes funds from program income, but excludes funding such as low-income housing tax credit funds (LIHTC) or non-Federal HOME match funds which are not consider housing assistance.

Determining the average cost of rehabilitation for the assisted units is calculated as follows:

Per Unit Rehabilitation $ = (a/c) + (b/d)

a= Federal Rehabilitation Assistance for all assisted units

b= Federal Rehabilitation Assistance for common areas and exterior painted surfaces

c= Number of federally assisted units

d= Total number of units     See 24 CFR 35.915.

Intent: Abatement, rehabilitation or weatherization?

Pursuant to a joint letter from HUD and EPA, dated April 19, 2002, jurisdictions have an additional option when rehabilitating dwellings in which LBP may be present.  This provision impacts dwellings below the $25,000 Federal rehabilitation assistance category.

If a jurisdiction’s “intent” is to rehabilitate or weatherize a dwelling without mitigating any LBP hazards, then it is not required to perform interim controls in the area to be rehabilitated.  Intent is shown in the work write up.  If only rehabilitation aspects are included in the write up, then the intent is to rehabilitate and not mitigate LBP hazards.  However, “work safe” practices must still be used.  If LBP mitigation measures are included in the work write up, then the intent is to mitigate LBP hazards, and all usual mitigation rules apply.

Additionally, if your intent is to abate LBP hazards, then you must follow the abatement work safe rules (discussed in Section III of this chapter) regardless of the hard cost level of your project.

CONSTRUCTION WASTE

According to a January 23, 2001 EPA Policy Guidance letter, #2001-02, “…all wastes generated from lead hazard control activities at residential properties are household wastes which are excluded from the hazardous waste requirements of the Resource Conservation and Recover Act.  As a result, residential LBP waste may be discarded in a municipal solid waste landfill or combustor, but not dumped nor open-burned.  Certain LBP waste (such as large quantities of concentrated waste—paint chips, dust, or sludge) from residential de-leading may be subject to more stringent State, local, and/or tribal requirements.”  As a result of this guidance, check with your local waste site to determine how they want to deal with the waste being generated from your LBP mitigation activities.

VII. Disclosures and Notices (24 CFR 35.92 and 24 CFR 35.125)

A, Disclosures

Notification is required on all rehabilitation and first time homebuyer activities, regardless of the level of assistance. Notification consists of:

- Lead Hazard Information Pamphlet:  Residents and purchasers of a residential property must receive a copy of the EPA/HUD/Consumer Product Safety Commission Lead Hazard Information Pamphlet, “How to Protect your Family from Lead in Your Home.”  Have the recipient acknowledge receipt of this pamphlet in writing and retain this acknowledgment in the rehabilitation files.

- Disclosure of LBP and Hazards:  Property owners must provide purchasers and lessees with available information or knowledge regarding the presence of LBP and hazards prior to selling or leasing a residence.  In the contract or lease, sellers and leasers of pre-1978 housing must include disclosure and acknowledgement language and a warning statement about the dangers of LBP.  Sellers must allow purchasers 10 days to inspect the dwelling for LBP or lead based paint hazards.  The appropriate contractual addendum on this waiting period must be part of the contract documents.

B.  Notice and Forms:

- Paint Testing, Lead Hazard Evaluation, Visual Assessment, or Presumption:  Use DHS form 8552 when an evaluation results in findings of LBP hazards or if a presumption of LBP hazards is made. Contractors must provide notice to the residents, and post notice no later than 15 days after this report has been received.  These notices shall be posted for four weeks.

- Abatement:  Use DHS form 8551, Abatement of Lead Hazards Notification in conjunction with DHS form 8552.

- Clearance:  Use DHS form 8552.  Notice of Lead Hazard Reduction Activity:  When lead hazard reduction activities have been completed, a contractor must provide to the residents or post a notice of these lead hazard reduction activities no later than 15 days after completion.  The notices shall be updated if additional work is required.  Any failed clearances must be described in the notice.  The notices shall be posted for four weeks.

- Availability of Notices:  Notices of evaluation, presumption, and hazard reduction shall:

- Be of a size and type easily read by occupants.

- To the extent practicable, be made available upon request in a format accessible to persons with disabilities (e.g., Braille, large type, computer disk, audio tape).

- Be provided in the occupants’ primary language or in the language of the occupants’ contract or lease.

- Be provided to the occupants by posting and maintaining it in centrally located common areas and distributing it to any dwelling unit, if necessary, because the head of household is a person with a known disability.

- Be distributed to each occupied dwelling affected by the evaluation.  If the notice is for a lead hazard in a common area that is used by those in the dwelling unit, the occupants of the dwelling unit shall also receive notice.

VIII.	ACQUISITION AND SUPPORT SERVICES (24 CFR 35.1000)

The lead-based paint requirements for acquisition, leasing, support services or operations is found in 24 CFR Part 35, Subpart K. This Section applies to both FTHB and Rental Housing Acquisition.

A.  Applicability

The HUD LBP disclosure requirements set forth in 24 CFR 35.80–35.98 apply to all transactions to sell or lease housing constructed prior to 1978, also referred to as “target housing”.

Acquisition Lead Requirements

These regulations are intended to provide assurances that the LBP paint in homes purchased with HOME funds have been stabilized, and that the unit is “lead safe” when it is occupied by the assisted household.  Key requirements for LBP in Homebuyer Assistance are LBP hazard evaluation, treatment, and clearance.  The following is required to identify deteriorated paint in homes:

- Visual Assessment.  An inspection of all interior painted surfaces, including common areas such as hallways, laundry rooms or garages, and exterior surfaces of the building in which the dwelling unit is located must be conducted to identify deteriorated paint.  Notification is only required if LBP hazards are identified.

- Paint Stabilization.  All deteriorated paint surfaces must be stabilized before the homebuyer moves into the home.  If paint testing of a deteriorated surface reveals no LBP, then paint stabilization is not required on that surface.

- Safe Work Practices.  The owner/contractor must use safe work practices when conducting paint stabilization.  Safe work practices include safe work methods, occupant protection, worksite preparation, and cleanup.

- Clearance.  After the completion of work, the home must pass clearance.  Clearance must happen before occupancy if the home is vacant or immediately after receipt of Federal assistance for a home currently occupied.

- Costs.  In order to provide maximum flexibility, the party responsible for paying for lead hazard evaluation and reduction in homebuyer programs depends upon program design and local requirements.  Costs may be borne by the administering agency, the seller, the homebuyer, or a combination of the above.  HOME provides LBP Grants, the cost of which is not included in the HOME Loan amount.

- Notification:  The notification process is the same as for rehabilitation activities.  (See Section VI of this chapter.)

B.  Responsibilities of Sellers and Lessors

The seller or lessor must do the following:

- Lead Information Pamphlet – provide the purchaser or lessee with an EPA-approved lead hazard information pamphlet.

- Disclosure – disclose to the purchaser or lessor the presence of any known LBP and/or LBP hazards in target housing, including such information as: the basis for this determination; the location of lead paint/hazards; and the condition of the painted surfaces.

Records and Reports

- The seller or lessor must also disclose the existence of any available records or reports pertaining to lead paint and/or lead paint hazards in the housing, and provide the purchaser or lessee with any of these records or reports available to the seller.

- This requirement applies to information pertaining to lead paint/lead hazards in common areas, as well as in other dwelling units within multifamily housing.

Post-Offer Disclosure

If any lead disclosure occurs after an offer to purchase or lease has been made, the seller or lessor shall:

- Complete the required lead disclosure activities prior to accepting the offer; and

- Allow the purchaser or lessee the opportunity to review the information provided at disclosure, and amend the offer.

Opportunity to Conduct Evaluation

The seller must allow the purchaser 10 days to conduct a risk assessment or inspection for lead paint/lead hazards before any obligation to purchase.

C.  Certification and Acknowledgement of Disclosure – Contract Language

Contracts to sell or lease target housing must have an attachment that contains the following:

- Lead Warning Statement – See 24 CFR 35.92 in Appendix VI-A of this chapter.

- Disclosure Statement – A statement by the seller/lessor disclosing the presence of known lead paint and/or lead paint hazards in the housing, or indicating that they have no such knowledge.  Also included should be any additional information regarding the basis for such determinations, the location of lead paint/lead paint hazards, and the condition of painted surfaces.

- Records and Reports Provided – The contract must also contain a list of records and reports available to the seller/lessor that have been provided to the purchaser/lessee regarding the existence of lead paint/lead paint hazards in the housing.  If no such records or reports are available, the contract shall indicate this.

- Confirmation – A statement by the purchaser/lessee confirming the receipt of all of the required disclosure information, and in sales transactions, confirming that he/she has either received an opportunity to conduct a risk assessment or lead inspection, or waived this opportunity.

- Agents – Where there is an agent acting on behalf of the seller/lessor, the contract must also contain a statement that the agent has informed the seller/ lessor of their obligations and the agent is aware of his/her duty to ensure compliance with applicable lead disclosure requirements.

- Signatures – All of the parties to the contract, including the agent, must sign and date the attachment, certifying the accuracy of their statements.

- Retention –The seller/lessor shall retain a copy of the completed contract attachment for at least three years from the date of the sale/commencement of the leasing period.

IX.  TENANT-BASED RENTAL ASSISTANCE

A. Applicability:  Children Under 6 – Lead hazard evaluation and reduction requirements under this section apply to:

- Dwelling units occupied or to be occupied by families or households that have children under age six,

- The common areas servicing such units,

- And the exterior painted surfaces associated with the unit and its common areas.

Common areas include areas through which residents pass to gain access to a unit, and other areas frequented by resident children less than six years old, including on-site play areas and child care facilities.

B.  Activities at Initial and Periodic Inspection

- Visual Assessment – Same as for acquisition projects. (Section VIII of this chapter.)

- Notice of Lead Hazard Evaluation or Presumption –- Same as for rehabilitation of less than $5,000 per unit. (Section VI of this chapter.)

- Pamphlet – Same as for acquisition projects. (Section VIII of this chapter.)

- Paint Stabilization

  - The owner is required to stabilize deteriorated paint in accordance with the provisions of 24 CFR 1330(a) and (b) before occupancy by a tenant receiving HOME rental assistance.

  - If such occupancy began prior to a periodic inspection, paint stabilization must be completed within 30 days of notifying the owner of the results of the visual assessment

  - Paint Stabilization includes repairs to any defect in the substrate that is causing the paint to deteriorate, removing loose paint and other material from the defective surfaces, and applying new protective coatings or paint.

- Clearance – Shall be performed in accordance with 24 CFR 35.1345. For relocation requirements during lead hazard reduction activities, see Appendix VI-I.

- Notice and Pamphlet – Same as for acquisition projects. (Section VIII of this chapter.)

7.   On-going Maintenance of Rental Properties – Same as for rehabilitation 	projects of less than $5,000.  (Section VI of this chapter)

C.  Requirement if a Child Has an Elevated Blood–Lead Level

Applies to HOME tenant-based rental assisted units when a child has been identified as having elevated blood-lead levels.  The following actions are required:

- Risk Assessment

  - Within 15 days after being notified by a public health department or other medical care provider that a child under six receiving tenant-based rental assistance has an elevated blood–lead level, the State Recipient shall do a risk assessment of the unit in which the child lived at the time the blood sample was taken, including the common areas servicing that unit.

  - If notified of an EBL child by a medical professional, the owner shall notify the public health department within 5 working days.

  - If notification of an EBL child is received from a source other than a medical provider or public health department, the State Recipient shall immediately verify this information with the public health department or medical provider before doing a risk assessment.

  - The risk assessment shall be conducted in accordance with 24 CFR 35.1320(b). The requirement to conduct a risk assessment still applies if another assisted household is now occupying the unit, or is expected to occupy the unit.  The results of the risk assessment shall be provided to the owner immediately upon its completion.

- Hazard Reduction

- The property owner shall complete lead abatement (24 CFR 35.1325), or interim control activities (24 CFR 35.1330), within 30 days of receiving the risk assessment report from the State Recipient, or an evaluation from the public health department.

- Hazard reduction is complete when clearance has been achieved (24 CFR 35.1340), or when the public health department certifies that the lead hazard reduction is complete.  Failure to complete these activities as required constitutes a Housing Quality Standards Violation.

- Notice to Residents – The owner shall notify building residents of any evaluation or lead hazard reduction activities. (See 24 CFR 35.125 (Appendix VI-A) and DHS Forms 8552 and 8551 (Appendix VI-E, & VI-F), for information on required content, format, and distribution of notices.)

- Data Collection and Record Keeping – At least quarterly, the owner shall attempt to coordinate with the public health department(s) in the jurisdiction to identify EBL children who may be receiving HOME TBRA, and perform lead hazard evaluation and reduction activities as required by this section

X.  RECORDKEEPING

Records applicable to ongoing LBP maintenance and/or reevaluation shall be kept and made available for review until at least three years after such activities are no longer required. It is recommended that lead-based paint records be kept indefinitely

.  The following records should be kept:

- Information on age of property, age of children living at property, existing information on Children’s blood lead levels, existing information on lead-based paint

- Inspection report or documentation of Visual Assessment

- Disclosure statement

- Clearance report

- All notifications

- Documentation of required certifications or training, and

- Documentation indicating receipt of the pamphlet

# .XI.  Critical Laws and Regulations Relating to LBP

If other Federal, State, tribal or local laws, ordinances, codes or regulations are applicable to a HOME activity, the more protective definition shall apply.  Following are critical state and federal laws and regulations pertaining to LBP hazards.

# Federal:

- 24 CFR Part 35 – Lead Based Paint Prevention in Certain Residential Structures (HUD)

- Toxic Substances Control Act Section 406 - Requirements of Hazard Education Before Renovation for Target Housing

- 40 CFR Part 745  – Identification of Dangerous Levels of Lead (EPA)

State:

- Title 17 Sec. 35000 – Accreditation, Certification, and Work Practices for LBP and Lead Hazards (DHS)

- CalOSHA Title 8 Sec. 1532.1 – Construction Safety Orders – Lead

- CalOSHA Title 8 Sec. 5194 – Hazardous Communication

- Civil Code 1102-1102.16 –Real Estate Lead Hazard Disclosure Requirements (HUD also has its own disclosure requirements)

- Proposition 65 – Safe Drinking Water and Toxic Enforcement Act of 1986

### Local:

- Check with your city or county for applicable local codes.

XII.	questions & Answers on Lead

The following questions were taken from past Contract Management Trainings, from the HUD Lead Safe Housing Rule website, and from other materials.

GENERAL QUESTIONS

Question:  In a single family residence owned and occupied by an elderly household, they can waive their right to relocation?

Answer:  Yes but if there are other persons living in the residence under the age of 62, those persons may not have the waiver option.  For example, a grandmother raising her grandchild would not likely be able to waive the right to relocate.  Additionally, you should make sure that the elderly person has been given all the information and truly wishes to stay.  The waiver must be in writing. A sample form is available in the Appendix of this chapter.

Question:  Why is it optional for elderly people to stay in a dwelling with LBP during work that will disturb surfaces?

Answer:  Elderly are not in a high risk group, which includes children under the age of 6 and women of childbearing age.  Due to requirements for safe work areas, it is possible for elderly persons to be able to stay without long-term damage to their health.   A secondary reason for allowing those over 62 years old to waive the right to relocation is the difficulty the elderly may have in moving, even if it is only temporary.

Question:  Do all the units in a building have to be relocated if lead work is to be done?

Answer:  No.  You could have a rental rehabilitation project in which there are, for example, 6 units being rehabbed.  All but one unit may have families occupying the complex.  The one unit could have an elderly man renting.  If he is over 62, he could waive his right to relocate. (Please see “Guidance on Relocation” in the Appendix of this chapter for additional help with relocation)

Question:  Can relocation benefits be in the form of a grant?

Answer:  Yes, in fact they must be in the form of a grant.

Question:  Is there a limit to the size of the grant for lead remediation?

Answer:  There isn’t a specific dollar limit, but the amount that can be granted is the minimum amount of work required to meet the lead paint requirements.  In other words, replacing all the windows wouldn’t be eligible for a grant.  But you could determine what the cost would be to do abatement and clearance of lead paint on the windows. The difference between the cost to replace the windows and the cost to abate the lead paint would be part of the loan amount.  For example, if window replacement costs $20,000 (this would remove the lead hazard), and the cost to abate lead paint on the existing windows is $10,000, then the grant would be $10,000 and the rehabilitation loan would be $10,000.

Question:  When do you have to “work safe?”

Answer: Always, unless the area to be worked on is less than the de minimis levels.

Question: How can I get information on certified training?

Answer: Certification training is done in California by The Department of Health Services. See: www.dhs.ca.gov/childlead/html/materials.html

Question: We need a list of certified LBP contractors in our area.

Answer: See: www.dhs.ca.gov/childlead/html/GENclist.html

Question: Who fills out the forms and notices?

Answer: For DHS forms, the certified risk assessor/inspector fills out forms #8551 and #8552 except in the case of visual assessments and presumptions.  Visual assessments can be filled out by someone who has completed the online HUD training The presumptions are an administrative task and may be filled out by any appropriate jurisdiction official. (See Appendix E and F of this chapter for copies of DHS Forms 8551 and 8552)

Question: Does a contractor certified in another state still have to take the California DHS certified test to work in State?

Answer: Yes. California does not recognize other states’ certifications.

Question: Who is responsible for fixing deteriorated paint identified during the visual inspection?

Answer: The Lead Safe Housing Rule does not specify who is responsible. The HOME Recipient may design its program to allow the seller, the purchaser, or a third party to make the repairs. However, the HOME Recipient must ensure that repairs are done using lead safe work practices. No special training is required in this case; however, the person who performs the work should certify that it was done in accordance with lead safe work practices.

hOMEBUYER ACTIVITY

Question:  If lead paint is found in a house that a first-time homebuyer applicant is purchasing, does the owner have to correct it before the house can be sold, or can it be sold “as is”?

Answer:  Under the HOME Program, they can not sell “as is” when lead hazards have been found or disclosed by the seller.  The Lead-Safe Housing Rule calls for, among other requirements, a visual assessment and paint stabilization when lead hazards are found.  HOME does allow for grants to remediate the hazard. And, the work can be done after a real estate settlement, but must be completed before occupancy.

Question: Who is responsible for fixing deteriorated paint identified during the visual inspection?

Answer: The Lead Safe Housing Rule does not specify who is responsible. The HOME Recipient may design its program to allow the seller, the purchaser, or a third party to make the repairs. However, the HOME Recipient must ensure that repairs are done using lead safe work practices. No special training is required in this case; however, the person who performs the work should certify that it was done in accordance with lead safe work practices.

Question:  The Jones family just purchased a home through the HOME funded downpayment assistance program from the ABC Housing Corporation. The ABC Housing Corporation did extensive rehabilitation on the unit prior to the purchase. Is the purchase subject to Subpart J or K?

Answer: Subpart K (Acquisition). The Jones family’s purchase did not trigger the rehabilitation. Any work done by ABC Housing Corporation was subject to the rules of its funding.

Question: What do we do if a homebuyer chooses to exercise her option to evaluate for lead hazards on a property?

Answer: All homebuyers, regardless of Federal assistance, have the option to evaluate for lead hazards on a property. Homebuyer programs should, therefore, be designed to account for this possibility, including contract language that allows for evaluation and policies to address any identified hazards.

Question:  If lead paint is found in a house that a first-time homebuyer applicant is purchasing, does the owner have to correct it before the house can be sold, or can it be sold “as is”?

Answer: Under the HOME Program, they can not sell “as is” when lead hazards have been found or disclosed by the seller.  The Lead-Safe Housing Rule calls for, among other requirements, a visual assessment and paint stabilization when lead hazards are found.  HOME does allow for grants to remediate the hazard. And, the work can be done after a real estate settlement, but must be completed before occupancy.

Question: If a city is to purchase a 4-plex and rehabilitate it, does the Housing Acquisition or Housing Rehabilitation LBP rules apply?

Answer: If any HOME (federal) funds are used for rehabilitation the more restrictive Rehabilitation rules (Subpart J) apply. For example, FTHB (Acquisition only) would be covered under Subpart K, but FTHB Acquisition with Minor Rehabilitation would be covered under Subpart J.

REHABILITATION ACTIVITY

- Question: An elderly woman applied for a loan under our owner-occupied rehab program. Is her unit exempt from the Lead Safe Housing Rule?

Answer: No. Only units that are designated for the elderly are exempt from the Lead Safe Housing Rule. See 35.115 (a)(3). However, because this unit is occupied by an elderly person, it is eligible for a relocation waiver that will allow the elderly woman to stay in the unit while work is being performed. Please see the Appendix of this chapter for an example of a waiver for the elderly.

Question: Does the Lead Safe Housing Rule apply to out-building such as garages, shed, or fences not attached to the residential building being rehabbed?

Answer:  Yes, the rule applies. Under Subpart B the definition of residential property is “a dwelling unit, common areas, building exterior surfaces, and any surrounding land, including outbuildings, fences, and play equipment affixed to the land, belonging to an owner and available for use by the residents” A HOME Recipient would be required to follow all steps that were taken for the residential building with any other structure on the property.

Question: Is relocation of occupants required for all jobs involving lead-based paint?

Answer: No. Relocation is only required when it is necessary to protect the occupants from lead hazards created during work. These conditions are defined at 24 CFR 35.1345.

Question: A job involves roof and furnace repair. The level of assistance is $6,000. The job will disturb a small amount of paint which is below the de minimis level. Is the unit exempt from the lead safe housing rule?

Answer: No. Because paint will be disturbed (even though it is a small amount), this unit is not exempt. Because the level of assistance is over $5,000, this job requires a risk assessment. If hazards are identified, they must be addressed using interim controls. Alternatively, you may presume that hazards are present and perform standard treatments on the whole unit.

- Question: Is my emergency repairs program exempt from the Lead Safe Housing Rule?

- Answer: No, the program is not exempt from the Lead Safe Housing Rule. However, individual emergency repair projects may be exempt from the Lead Safe Housing Rule requirements if they meet the exemption criteria defined at 35.115(a)(9) for emergencies (i.e., the condition to be corrected must require an “emergency action necessary to safeguard against imminent danger to human life, healthy or safety or to protect property from further structural damage.”) A good rule of thumb is that if the condition is not urgent enough to require action within the next 24 to 48 hours, it is not an emergency.

Question:  When doing owner-occupied rehabilitation, if the owner doesn’t want to stay in the house, we are required to pay relocation benefits?

Answer:  Yes, temporary relocation benefits.

Question:  But if there’s no lead based paint in the house, you don’t have to pay relocation benefits, right?

Answer: If an assessment has been conducted and it is determined no lead hazards have been found, then no relocation is needed and no benefits would be paid.

Question: How does sweat equity come into play when a homeowner opts to paint the house as part of the rehabilitation?  What is the homeowner’s responsibility in minimizing LBP hazards?

Answer: The homeowner must receive training (or already be certified) in and use work safe requirements, including containment, practices, and clearance.  This chapter and the Appendix (“Summary of Lead Training Requirements) will be useful to your program in determining if an individual homeowner has the qualifications to do LBP work themselves.

Question: When is abatement required?

Answer: When the intent is to permanently eliminate the LBP hazard (abate), and when the Federal Rehabilitation Assistance (FRA) is over $25,000 per unit.

tENANT BASED RENTAL ASSISTANCE ACTIvity

Question:  In tenant-based rental assistance, what is required to comply with the lead paint requirements?

Answer:  If the unit was built before 1978, the requirements are:  notification, visual assessment, paint stabilization, safe work practices, clearance, and on-going maintenance.  Additionally, the jurisdiction and property owners must take specific steps after receiving notice that a child with Environmental Intervention Blood Lead Levels (EIBBL) is living in a unit receiving TBRA. Please see the Appendix of this chapter for a useful chart that breaks down requirement by activity: “Summary of Lead-Based Paint Requirements by activity.”

XIII. Definitions

Abatement:  Any set of measures designed to permanently eliminate lead-based paint or lead-based paint hazards (see definition of ``permanent''). Abatement includes the removal of lead-based paint and dust-lead hazards, the permanent enclosure or encapsulation of lead-based paint, the replacement of components or fixtures painted with lead-based paint, and the removal or permanent covering of soil-lead hazards.

Bare soil:  Soil or sand not covered by grass, sod, other live ground covers, wood chips, gravel, artificial turf, or similar covering.

Certified:  DHS licensed or certified to perform such activities as risk assessment, lead-based paint inspection, or abatement supervision.

Chewable surface:  An interior or exterior surface painted with lead-based paint that a young child can mouth or chew. A chewable surface is the same as an “accessible surface'' as defined in 42 U.S.C. 4851b (2)). Hard metal substrates and other materials that cannot be dented by the bite of a young child are not considered chewable.

Clearance Examination:  An activity conducted following lead-based paint hazard education activities to determine that the hazard reduction activities are complete and that no soil-lead hazards or settled dust-lead hazards exist in the dwelling unit or worksite.

Common Area:  A portion of a residential property that is available for use by occupants of more than one dwelling unit. Such an area may include, but is not limited to, hallways, stairways, laundry and recreational rooms, playgrounds, community centers, on-site day care facilities, garages and boundary fences.

Containment:  The physical measures taken to ensure that dust and debris created or released during lead-based paint hazard reduction are not spread, blown or tracked from inside to outside of the worksite.

Dwelling Unit:  (1) Single-family dwelling, including attached structures such as porches and stoops; or (2) Housing unit in a structure that contains more than 1 separate housing unit, and in which each such unit is used or occupied, or intended to be used or occupied, in whole or in part, as the home or separate living quarters of 1 or more persons.

Encapsulation:  The application of a covering or coating that acts as a barrier between the lead-based paint and the environment. Encapsulation may be used as a method of abatement if it is designed and performed so as to be permanent (see definition of ``permanent'').

Enclosure: The use of rigid, durable construction materials that are mechanically fastened to the substrate in order to act as a barrier between lead-based paint and the environment. Enclosure may be used as a method of abatement if it is designed to be permanent (see definition of ``permanent'').

Evaluation:  A risk assessment, a lead hazard screen, a lead-based paint inspection, paint testing, or a combination of these to determine the presence of lead-based paint hazards or lead-based paint.

Federally Rehabilitation Assistance:  The amount used to determine which rehabilitation category used to address LBP hazards.  This figure is calculated by taking the lower of the total federal assistance in a dwelling and the total hard costs to rehab the dwelling.

Hazard reduction: Measures designed to reduce or eliminate human exposure to lead-based paint hazards through methods including interim controls or abatement or a combination of the two.

HEPA vacuum:  A vacuum cleaner device with an included high-efficiency particulate air (HEPA) filter.

Housing for the Elderly:  Retirement communities or similar types of housing reserved for households composed of one or more persons 62 years of age or more or other age if recognized as elderly by a specific Federal housing assistance program.

Impact Surface:  An interior or exterior surface that is subject to damage by repeated sudden force, such as certain parts of door frames.

Interim Controls:  A set of measures designed to reduce temporarily human exposure or likely exposure to lead-based paint hazards. Interim controls include, but are not limited to, repairs, painting, temporary containment, specialized cleaning, clearance, ongoing lead-based paint maintenance activities, and the establishment and operation of management and resident education programs.

Lead-Based Paint:  Paint or other surface coatings that contain lead equal to or exceeding 1.0 milligram per square centimeter or 0.5 percent by weight or 5,000 parts per million (ppm) by weight.

Lead-Based Paint Hazard:  Any condition that causes exposure to lead from dust-lead hazards, soil-lead hazards, or lead-based paint that is deteriorated or present in chewable surfaces, friction surfaces, or impact surfaces, and that would result in adverse human health effects.

Lead Hazard Information Pamphlet:  “How to Protect Your Child From Lead in Your Home.”  The HUD pamphlet used to notice a resident who is about to participate in a LBP activity.

Lead Hazard Reduction Activity:  The activity chosen to address the existence of LBP and/or LBP hazards.

Lead Hazard Screen:  A limited risk assessment activity that involves paint testing and dust sampling and analysis.  If lead hazards are found, then a full risk assessment is required.

Multifamily property:  A residential property containing five or more dwelling units.

Noticing:  Notifying the occupants (or potential occupants) of a dwelling of LBP related actions or history pertaining to that dwelling.  This noticing may take the form of a pamphlet or posted notices regarding LBP hazards found, treated, and cleared in a dwelling.

Paint Stabilization:  Repairing any physical defect in the substrate of a painted surface that is causing paint deterioration, removing loose paint and other material from the surface to be treated, and applying a new protective coating or paint.

Paint Testing:  The process of determining, by a certified lead-based paint inspector or risk assessor, the presence or the absence of lead-based paint on deteriorated paint surfaces or painted surfaces to be disturbed or replaced.

Permanent:  An expected design life of at least 20 years.

Play Area:  An area of frequent soil contact by children of less than 6 years of age, as indicated by the presence of play equipment (e.g. sandboxes, swing sets, sliding boards, etc.) or toys or other children's possessions, observations of play patterns, or information provided by parents, residents or property owners.

Presumption:  An administrative decision to presume LBP is present.  At times, this presumption may not require a risk assessment.

Risk Assessment:  An on-site investigation to determine the existence, nature, severity, and location of lead-based paint hazards; and the provision of a report by the individual or firm conducting the risk assessment explaining the results of the investigation and options for reducing lead-based paint hazards.

Risk Assessor/Inspector: This person performs risk assessments and clearance examinations.  This person must take and pass the 40 hour Risk Assessor/Inspector class.  DHS registration AND testing is required.

Safe Work Practices:  A system of working to remove LBP that minimizes spreading LBP dust and debris which would contaminate the workers and residents of a dwelling.

Single Family Property:  A residential property containing one through four dwelling units.

Single Room Occupancy (SRO) Housing:  Housing consisting of zero-bedroom dwelling units that may contain food preparation or sanitary facilities or both (see Zero-bedroom dwelling).

Soil-Lead Hazard:  Bare soil on residential property that contains excessive amounts of lead.

Standard Treatments:  A series of hazard reduction measures designed to reduce all lead-based paint hazards in a dwelling unit without the benefit of a risk assessment or other evaluation.

Substrate:  The material directly beneath the painted surface out of which the components are constructed, including wood, drywall, plaster, concrete, brick or metal.

Supervisor:  This person supervises certified and non-certified LBP workers. This person must have taken and passed the 40 hour LBP Supervisor class. DHS registration AND testing is required.

Visual Assessment: Looking for deteriorated paint, visible surface dust, debris and residue which may be a part of a risk assessment or clearance examination.  A person must have taken the HUD VA test on the web.  www.hud.gov/offices/lead/training/visualassessment/h00100.htm

Worker:  There are two types of workers, certified and non-certified.

Certified:  A person who has taken the DHS, 3-day, 24 hour certified LBP certified worker class.  While no DHS test is required, DHS registration is required.  This person may work on any LBP project, but only under supervision.

None Certified:  A person who has taken the HUD 1 day, 8 hour “Work Safe” class.  This worker may not work on abatement projects.